25 pins connector to connect High Voltage PIEZO signals and thermometer

| 1 | thermometer PIN 3 | 14 |  |
| --- | --- | --- | --- |
| 2 | thermometer PIN 5 | 15 |  |
| 3 | thermometer PIN 1 | 16 |  |
| 4 | thermometer PIN 4 | 17 |  |
| 5 | Common | 18 |  |
| 6 | C3? | 19 | Z |
| 7 | Common | 20 |  |
| 8 | C2 | 21 |  |
| 9 |  | 22 |  |
| 10 | C1 | 23 |  |
| 11 | Y | 24 |  |
| 12 |  | 25 |  |
| 13 | X |  |  |

22/12/2010, new wiring:

Pin 5 & 7 to common.

Pin 1/2/3/4 to DIN thermometer

Pin 11 Y piezo

Pin 13 X Piezo

Pin 19 Z piezo

Wiring:

4* 5 mtr coax kabel,

1* 5 mtr 4-wire cable for thermometer. All cables are bundled, except for last meter on side of BNC.

Connectoren:

1* female 25 pins connector naar 4*male BNC connectoren en 5 pins DIN

Shielding:

Shield of BNC wires only connected at bnc plugs

Shield of Temperature wire only connected at DIN plug (PIN 3)

Marius Trouwborst, telefoon: 0614886859

Wiring FEMTO IV-Convertor

Color coding 25 pins connector

Pin

9: grijs GND

10: wit   DI 0

11: geel DI 1

12: groen  DI 2

13: bruin DI 3

14: geel DI 4