Federica Galli

Leiden Institute of Physics

Niels Bohrweg 2

2333 CA Leiden

The Netherlands

Tel.:	+31 71 5275404

27th December 2006

Quotation No Q5588

Helium level probe

The sensor consists of a fine wire made from an alloy (NbTi) which has a superconducting transition temperature higher then the boiling point of liquid helium. A small heater wound on the sensor wire is used to ensure that a short section at the top of the wire remains resistive independent of the liquid helium level. The voltage developed across the sensor is then directly proportional to the length of conductor in the helium vapour. This voltage is internally converted (in the HLG200 or HLG210) to equivalent helium depth and digitally displayed. This operation is pulsed intermittently to minimise heat input into the helium but the level is displayed continuously.

Type 105

This probe is suited for measuring helium depth to a maximum of 550mm and is available in total lengths from 600mm to 2000mm, with 1100mm as our standard stock length.

##### Prices, Terms and Conditions

| Item | Description | Price |
| --- | --- | --- |
| HLP-AL550-TL1100 | Helium level probe (type 105A) with active length 550mm and total length 1100mm. | £215 |
| HLP-AL550-TLXX | Helium level probe (type 105B) with active length 550mm and total length 600mm-1100mm. | £210 |
| HLP-AL550-TLXX | Helium level probe (type 105C) with active length 550mm and total length 1100mm-2000mm. | £235 |

- The prices above are quoted as ex-works London, exclusive of packing, freight and insurance. They also exclude VAT (Value Added Tax); please indicate your VAT number on your order

| Packing & insurance for one level probe | £ 25 |
| --- | --- |
| Delivery for one level probe | £ 30 |

| PAYMENT TERMS | 100% net on delivery |
| --- | --- |
| DELIVERY | Typically 2 weeks from receipt of written order. |
| VALIDITY OF OFFER | 90 days |
| WARRANTY | All items are covered by our standard warranty of one year against defects in materials or faulty workmanship. |
| LOCAL TAXES AND/OR DUTIES | Prices exclude local taxes and/or import duty. |

| Felicia Doughty |  |
| --- | --- |
| Sales Administrator |  |